Job Aid 4: Proper Inventory Storage Guidelines301

| Identify a secure and adequate storage site |  | Organize the supplies carefully |
| --- | --- | --- |
| - Locked - Accessible only to authorized personnel - Free from extreme temperature and humidity - Free from direct sunlight exposure - Free of pests - Free from excess moisture (water leaks and drips) - Free of clutter and trash - Adequate ventilation - Sufficient lighting |  | - Use shelves and bins to organize supplies - Store according to temperature requirements - Store similar items together (controls with controls, calibrators with calibrators) - Group identical items in smaller groups that are easy to count - Arrange items within each group by alphabetical order - Store all items on shelves (not on the floor) - Label the shelves with the name of each item in that area of the shelf - Perform inventory management of supplies and reagents - Store all items on shelves with shorter expiry dates at the front (FEFO) - Rotate stock (FIFO) - Check for any expired reagents/supplies - Designate, where appropriate, an area or on the items themselves: - Received, not yet evaluated - Evaluated, ready for use - Not acceptable for use, to be returned or disposed |
| Assess storage requirement as indicated by manufacturer for each reagent and supply |  |  |
| - Keep ambient supplies in designated well maintained and monitored room (record temperature daily) - Keep refrigerated supplies in designated well maintained and monitored refrigerators (record temperature daily) - Keep frozen supplies in designated well maintained and monitored freezer (record temperature daily) |  |  |
| Ensure safety of storage area |  |  |
| - Appropriate storage of hazardous chemicals according to MSDS - Glass or breakable items are stored on lower shelves - All items are properly identified and labeled |  |  |