Job Aid 1: PT General Guidelines 609

General Guidelines for Proficiency Testing

Laboratory proficiency testing (PT) is an essential element of laboratory quality assurance. Proficiency testing is an independent and unbiased assessment of the performance of all aspects of the laboratory, both human and hardware.  Proficiency testing provides an assessment of the validity of testing in your laboratory.

Handling Your PT Survey

Pre-analytical

- Note the date of receipt for your shipment

- Immediately inspect and reconcile the contents of your shipment with the accompanying paperwork

- Are all required specimens available?

- Is the quality and appearance of the specimens acceptable?

- Store the shipment properly

- Note due date of results

- Reconstitute specimens with volumetric pipettes and correct diluent

- Mix samples well before analyzing

Analytical

- Analyze specimens at correct temperature.  If shipment was stored in the refrigerator, specimens may need to come to room temperature before testing.

- Always refer to your survey instructions for storage and specimen handling.

- Analyze PT specimens in the same fashion as patient specimens

- Rotate testing responsibility for PT specimens between all laboratory personnel that are routinely performing the analysis in your laboratory

- Perform PT analysis well before due date of results

Post-Analytical

- Assure that your laboratory’s results are reported according to the PT provider’s instructions

- Review results for clerical errors on answer sheet

- Retain a copy of answer sheet for your records.  Attach all raw data and the instrument print-out to the answer sheet.

- If possible, retain specimens in freezer for confirmatory testing if needed

Receipt of Results

- Review your results with your peer grouping

- Have Laboratory Director and Supervisor review and sign results

- Review results with testing personnel.  Retain a copy for competency assessment and place into personnel record

- Investigate any failed responses and complete an EQA Failure Checklist assessment.

- Follow-up with remedial actions if indicated